# ALFA Field: Data Request for Waterflood Injection Optimization

Requested by: ________________________    Date: 2026-09-07    Requested return date: ____________

To: Field surveillance / production engineering / reservoir engineering, ALFA

Attachment: ALFA_data_request_template.xlsx (one sheet per table, headers and units pre-filled)

# 1. Purpose

We are evaluating injector-producer connectivity and injection allocation for the field with a capacitance-resistance (CRM) workflow. A first pass on the monthly allocated production and injection history (2008-06 to 2026-07, 48 wells) ran successfully but gave LOW confidence in every configuration: the field behaves as a single tank in the data we have, so no per-injector recommendation can be defended yet.

The limitation is information content, not the file format. Three things would change the outcome: flowing bottom-hole pressure on producers, higher-frequency rates from the surveillance system instead of monthly allocation, and a period in which injection rates are changed deliberately. This request lists what we need, in priority order, and how to send it.

# 2. Summary of requested items

| # | Item | Priority | Wells | Period | Frequency | Owner / status |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Producer flowing bottom-hole pressure | Essential | All producers (priority: ALFA-33, ALFA-08, ALFA-35, ALFA-09, ALFA-28, ALFA-27) | 2020-01 to present (older if available) | Daily preferred; monthly minimum |  |
| 2 | Producer well tests (surveillance rates) | Essential | All producers | 2020-01 to present | Every test (typically weekly to monthly) |  |
| 3 | Daily metered injection per well | Essential | All injectors (priority: ALFA-41, ALFA-02, ALFA-46, ALFA-43) | 2020-01 to present | Daily |  |
| 4 | Static reservoir pressure surveys | High | All wells | Whole field life | Every survey |  |
| 5 | Well event history | High | All wells | 2008-06 to present | Every event |  |
| 6 | Injection step test on the 41 to 46 cluster (proposal) | High | ALFA-41, ALFA-02, ALFA-46, ALFA-43 with daily tests on ALFA-33, ALFA-08, ALFA-35, ALFA-09, ALFA-28, ALFA-27 | Planned: 2 to 3 months | Daily during the test |  |
| 7 | Well master data | Useful | All wells | Current | Once |  |
| 8 | Reservoir and fluid properties | Useful | Field | Current | Once |  |
| 9 | Monthly allocation notes (already received data) | Useful | All wells | 2008-06 to 2026-07 | Once |  |

Priority: Essential = the study cannot progress beyond screening without it; High = materially improves the match; Useful = improves interpretation and unit conversions.

# 3. Item details

## 1. Producer flowing bottom-hole pressure  (Essential)

Wells: All producers (priority: ALFA-33, ALFA-08, ALFA-35, ALFA-09, ALFA-28, ALFA-27).   Period: 2020-01 to present (older if available).   Frequency: Daily preferred; monthly minimum.

What we need: Flowing BHP from permanent gauge, ESP intake pressure (PIP) or survey. If only wellhead pressure exists, send WHP together with fluid level or the calculated BHP and the method used. Include gauge / intake depth and the datum used.

Why it matters: Without flowing pressure the model must assume constant BHP and reads every choke, lift or shut-in change as reservoir response. This is the single largest limitation in the current match.

## 2. Producer well tests (surveillance rates)  (Essential)

Wells: All producers.   Period: 2020-01 to present.   Frequency: Every test (typically weekly to monthly).

What we need: Test date and duration, oil, water and gas rates during the test, choke size, hours on, lift type and frequency / gas-lift rate, and whether the test was accepted for allocation.

Why it matters: Monthly allocated volumes cannot resolve response times below about a month. Well tests give 10 to 50 times more information per model parameter.

## 3. Daily metered injection per well  (Essential)

Wells: All injectors (priority: ALFA-41, ALFA-02, ALFA-46, ALFA-43).   Period: 2020-01 to present.   Frequency: Daily.

What we need: Daily metered injection rate per well (not header allocation), wellhead injection pressure, hours injecting, and meter type / last calibration date. Flag days where the meter was down and the value is estimated.

Why it matters: Injection is the input signal of the model. Allocated monthly volumes booked against a few operating days create false rate spikes.

## 4. Static reservoir pressure surveys  (High)

Wells: All wells.   Period: Whole field life.   Frequency: Every survey.

What we need: Survey date, well, survey type (SBHP, build-up, fall-off, RFT/MDT), measured pressure and depth, pressure corrected to datum, datum depth, interpreted p* where available.

Why it matters: Constrains aquifer strength and voidage balance; today the field-tank model absorbs both.

## 5. Well event history  (High)

Wells: All wells.   Period: 2008-06 to present.   Frequency: Every event.

What we need: Date, well and type for: workover, conversion (producer to injector or back), recompletion, infill / new perforation, stimulation, ESP change or resize, choke change, extended shut-in, water shut-off. One line per event with a short note.

Why it matters: Lets the model explain rate steps as operations rather than fitting them as reservoir behaviour.

## 6. Injection step test on the 41 to 46 cluster (proposal)  (High)

Wells: ALFA-41, ALFA-02, ALFA-46, ALFA-43 with daily tests on ALFA-33, ALFA-08, ALFA-35, ALFA-09, ALFA-28, ALFA-27.   Period: Planned: 2 to 3 months.   Frequency: Daily during the test.

What we need: Hold each injector at two or three rate levels for 2 to 4 weeks each, changing one injector at a time where possible, with daily well tests and flowing BHP on the six producers. Please send any planned or historical rate-change tests already carried out.

Why it matters: The model identifies connectivity from rate changes. Current injection is on/off with few deliberate changes, so the signal is weak.

## 7. Well master data  (Useful)

Wells: All wells.   Period: Current.   Frequency: Once.

What we need: Surface and bottom-hole coordinates (state metres or feet), reservoir / layer each well is completed in, perforated intervals, completion date, well type history, pattern assignment.

Why it matters: Confirms the injector-producer pairs that are physically possible and the distance weighting.

## 8. Reservoir and fluid properties  (Useful)

Wells: Field.   Period: Current.   Frequency: Once.

What we need: Bo, Bw, Rs, oil and water viscosity at reservoir conditions, rock and fluid compressibility, average porosity and net thickness, original and current reservoir pressure, aquifer description, current voidage replacement ratio as reported by the asset.

Why it matters: Needed to convert the model's time constants into pore volumes and to check the voidage balance.

## 9. Monthly allocation notes (already received data)  (Useful)

Wells: All wells.   Period: 2008-06 to 2026-07.   Frequency: Once.

What we need: Description of the allocation method (test-based, meter-based, back-allocated from a header), which months are partial or estimated, and what the injection day count means when it is blank or below 5 days while a full monthly volume is booked.

Why it matters: Explains the 323 injection months in the received file that have volume but no usable day count, and the July 2026 partial month.

# 4. Data format rules

One row per well per date. Use a blank cell, not zero, when a value was not measured.

Well identifiers exactly as in the well master list (for example ALFA-02), without suffixes such as ' WI' and without spaces before or after.

Dates as ISO text (YYYY-MM-DD) or true Excel dates. State the time zone if daily data cross midnight.

Put the unit in every column header (for example fbhp_psi, q_oil_bbl_d, x_m). Do not mix units within a column.

Send rates as measured or as calendar-day rates. Do not divide monthly volumes by operating days.

Keep raw values. If a value is estimated, calculated or allocated, say so in a 'source' or 'flag' column instead of altering the number.

CSV or Excel is preferred, one file per table using the attached workbook sheets. Parquet or a database extract is also fine.

Do not remove outliers or shut-in periods; the study needs them to detect events.

# 5. Column layout per table

The attached workbook has these sheets with headers, units and one example row. Extra columns are welcome; please keep the listed ones.

### Sheet 01_flowing_pressure

Producer flowing bottom-hole pressure. One row per well per date (daily preferred).

| Column | Unit / allowed values | Example |
| --- | --- | --- |
| well | text, as in well master | ALFA-33 |
| date | YYYY-MM-DD | 2025-03-14 |
| fbhp_psi | psi (flowing BHP at gauge) | 1850 |
| whp_psi | psi (wellhead) | 210 |
| gauge_depth_m | m TVD (gauge / intake) | 1420 |
| datum_depth_m | m TVD (if corrected) | 1500 |
| source | gauge ¦ PIP ¦ survey ¦ calculated ¦ fluid_level | PIP |
| flag | measured ¦ estimated ¦ suspect | measured |
| note | free text |  |

### Sheet 02_well_tests

Producer well tests. One row per test.

| Column | Unit / allowed values | Example |
| --- | --- | --- |
| well | text | ALFA-09 |
| test_date | YYYY-MM-DD | 2025-03-14 |
| duration_h | hours | 6 |
| q_oil_bbl_d | bbl/d during the test | 180 |
| q_water_bbl_d | bbl/d | 620 |
| q_gas_mscf_d | Mscf/d | 95 |
| choke_64th | 1/64 in | 32 |
| hours_on_day | hours the well produced that day | 24 |
| lift_type | natural ¦ ESP ¦ gas_lift ¦ rod_pump ¦ PCP | ESP |
| lift_setting | Hz or gas-lift Mscf/d | 52 |
| fbhp_psi | psi if measured during the test | 1830 |
| accepted | yes ¦ no | yes |
| note | free text |  |

### Sheet 03_injection_daily

Daily metered injection per injector. One row per well per day.

| Column | Unit / allowed values | Example |
| --- | --- | --- |
| well | text | ALFA-41 |
| date | YYYY-MM-DD | 2025-03-14 |
| q_inj_bbl_d | bbl/d metered | 650 |
| hours_inj | hours injecting that day | 24 |
| whip_psi | psi wellhead injection pressure | 1450 |
| meter | well_meter ¦ header_allocated | well_meter |
| flag | measured ¦ estimated ¦ meter_down | measured |
| note | free text |  |

### Sheet 04_static_pressure

Static reservoir pressure surveys. One row per survey.

| Column | Unit / allowed values | Example |
| --- | --- | --- |
| well | text | ALFA-08 |
| survey_date | YYYY-MM-DD | 2024-11-02 |
| survey_type | SBHP ¦ build_up ¦ fall_off ¦ RFT_MDT ¦ other | build_up |
| shut_in_h | hours shut in before measurement | 72 |
| p_measured_psi | psi at gauge | 2210 |
| gauge_depth_m | m TVD | 1480 |
| p_datum_psi | psi corrected to datum | 2235 |
| datum_depth_m | m TVD | 1500 |
| p_star_psi | psi extrapolated (if interpreted) | 2260 |
| note | free text |  |

### Sheet 05_events

Well events. One row per event.

| Column | Unit / allowed values | Example |
| --- | --- | --- |
| well | text | ALFA-46 |
| date | YYYY-MM-DD | 2022-06-15 |
| type | WORKOVER ¦ CONVERSION ¦ RECOMPLETION ¦ INFILL ¦ STIMULATION ¦ ESP_CHANGE ¦ CHOKE_CHANGE ¦ SHUT_IN ¦ WATER_SHUTOFF ¦ OTHER | CONVERSION |
| note | free text (what changed, from / to) | producer to water injector |

### Sheet 06_well_master

Well master data. One row per well.

| Column | Unit / allowed values | Example |
| --- | --- | --- |
| well | text | ALFA-02 |
| x_m | m easting (state CRS in note) | 512345.0 |
| y_m | m northing | 2987654.0 |
| x_bh_m | m easting at mid-perforation |  |
| y_bh_m | m northing at mid-perforation |  |
| reservoir | text (layer / unit) | Upper A |
| top_perf_m | m TVD | 1490 |
| base_perf_m | m TVD | 1512 |
| completion_date | YYYY-MM-DD | 2009-04-01 |
| current_type | producer ¦ injector ¦ shut_in ¦ abandoned | injector |
| pattern | text | P-41 |
| platform_manifold | text (shared facilities) | WHP-3 |
| note | free text |  |

### Sheet 07_reservoir_fluid

Reservoir and fluid properties. One row per property.

| Column | Unit / allowed values | Example |
| --- | --- | --- |
| property | text | Bo |
| value | number | 1.25 |
| unit | text | rb/stb |
| condition | text (pressure, temperature, date) | at current p_res |
| source | text (PVT report, simulation, estimate) | PVT report 2015 |

# 6. Questions for the field team

Which producers have permanent downhole gauges or ESP intake pressure, and since when?

Are injection rates metered per well or allocated from a header meter?

How are the monthly injection day counts recorded, and what does a blank count mean?

Have any injection rate-change tests, interference tests or tracer surveys been done on the 41 to 46 cluster?

Which wells share a wellhead platform or manifold, so that a choke change on one affects another?

Is there an aquifer or pressure-maintenance study we should align with?

# 7. Proposed injection step test (item 6)

Cluster: injectors ALFA-41, ALFA-02, ALFA-46, ALFA-43; observed producers ALFA-33, ALFA-08, ALFA-35, ALFA-09, ALFA-28, ALFA-27. This group is the only part of the field where an allocation model currently beats the tank model, so it is the best place to prove the method.

Baseline: 2 weeks at current rates with daily well tests and flowing BHP on the six producers.

Step 1: raise one injector by 30 to 50 % for 3 weeks, others unchanged. Record daily rates and pressures.

Step 2: return to baseline for 2 weeks, then repeat for the next injector.

Where operations allow, one injector shut in for 2 weeks gives the clearest signal.

Total 2 to 3 months. Injection targets, facility limits and any safety constraints to be agreed with operations before start.

# 8. Delivery

Send files to: ______________________. Essential items first, as they become available; partial history is useful immediately. Please name files <field>_<sheet>_<YYYYMMDD>.csv or return the workbook with the sheets filled in. Confidential data will be kept outside any shared repository.

Contact for questions: ______________________