# Waterflood injection recommendation

streak_5x4 sample · asset SAMPLE · unit system field

Confidence: HIGH

Objective: maximum NPV · posture: balanced · horizon: 24 months · generated 2026-09-04 08:22 UTC by admin

# Sector S1 — summary

| item | value |
| --- | --- |
| confidence | HIGH — blind R² 1.00; parameter spread 0%; no gate warnings |
| method | CRMP (producer) |
| blind-test R² / MAPE | 1.000 / 0.0 % |
| gain vs hold-current | +1.1 % |
| gain vs equal split | +1.3 % |
| plan value (P10 / P50 / P90) | 24,393 / 24,393 / 24,393 bbl |
| hold-current (P10 / P50 / P90) | 23,925 / 23,925 / 23,925 bbl |
| changes this week | 5 |
| posture | balanced |

## Change list

| well | from [bbl/d] | to [bbl/d] | step this week | target date | setting hint | expected oil [bbl] | revert if |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I-3 | 1,841 | 0 | -276 | 2023-01-15 | target 0; set choke / pump frequency to reach it, field to confirm the setting | 124 | water cut of any connected producer rises by more than 5% |
| I-4 | 1,646 | 0 | -247 | 2023-01-08 | target 0; set choke / pump frequency to reach it, field to confirm the setting | 111 | water cut of any connected producer rises by more than 5% |
| I-1 | 2,310 | 3,612 | 346 | 2019-12-29 | target 3612; set choke / pump frequency to reach it, field to confirm the setting | 87 | water cut of any connected producer rises by more than 5% |
| I-5 | 1,450 | 2,695 | 217 | 2020-01-05 | target 2695; set choke / pump frequency to reach it, field to confirm the setting | 84 | water cut of any connected producer rises by more than 5% |
| I-2 | 1,782 | 2,722 | 267 | 2019-12-29 | target 2722; set choke / pump frequency to reach it, field to confirm the setting | 63 | water cut of any connected producer rises by more than 5% |

## Connectivity

## Forecast and injector efficiency

## Which method won

| gate | result |
| --- | --- |
| history_min | pass |
| od | pass |
| points_per_param | pass |
| crmip_data | pass |
| inj_cv | pass |
| tau_dt | pass |
| influx | fail / not applicable |
| event_heavy | fail / not applicable |
| mature | pass |
| long_history | pass |
| has_xy | pass |
| has_bhp | fail / not applicable |
| has_relperm | fail / not applicable |

## History match per well

## Sampling and pressure

Δt = 30 d · τ estimate = 125 d · τ/Δt = 4.1 · O_d = 20.0 · pressure source: none

# Data quality

Traffic light: green · steps: 120 · outliers removed: 0 · simultaneous P+I steps: 0 · unmatched IDs: 0 · warnings: 0 · errors: 0

| well | type | coverage % | pressure | conversions |
| --- | --- | --- | --- | --- |
| I-1 | INJ | 100 |  |  |
| I-2 | INJ | 100 |  |  |
| I-3 | INJ | 100 |  |  |
| I-4 | INJ | 100 |  |  |
| I-5 | INJ | 100 |  |  |
| P-1 | PROD | 100 | none |  |
| P-2 | PROD | 100 | none |  |
| P-3 | PROD | 100 | none |  |
| P-4 | PROD | 100 | none |  |

# Conditions (plain language)

| severity | what it means | suggested action | scope |
| --- | --- | --- | --- |
| info | Reservoir-volume conversion uses constant formation-volume factors (Bo = 1.0, Bw = 1.0). | Load a PVT table if formation volume factors change with pressure. | field |
| info | No flowing pressure data; lift or choke changes must be in the events table. | Load ESP/gauge pressure or an events log if lift settings changed during the history. | field |
| info | The MPI (geometric) model would suit this data but is not available in this release. | None needed; the next best available model was used. | field |
| info | The MLR / BMLR model would suit this data but is not available in this release. | None needed; the next best available model was used. | field |
| info | The PINN / residual ML model would suit this data but is not available in this release. | None needed; the next best available model was used. | field |
| info | The model misses some slow trends for P-1; forecasts may drift. | None needed for screening; a residual correction is considered when history exceeds 120 months. | well:P-1 |
| info | The model misses some slow trends for P-2; forecasts may drift. | None needed for screening; a residual correction is considered when history exceeds 120 months. | well:P-2 |
| info | The model misses some slow trends for P-3; forecasts may drift. | None needed for screening; a residual correction is considered when history exceeds 120 months. | well:P-3 |
| info | The model misses some slow trends for P-4; forecasts may drift. | None needed for screening; a residual correction is considered when history exceeds 120 months. | well:P-4 |

# Workflow

| item | value |
| --- | --- |
| recommendation | 422b38fb649a |
| state | APPROVED |
| originator | admin |
| approved by originator | YES — same person ran and approved |
| override reason | — |
| advanced overrides | none |
| snapshot digest | 6a0b517bad5e5f2fa974da02b1b8e8ce338281d5c7e683826a350295d6d61cc6 |
| implemented at | — |

| when | action | actor | acting role | from → to | note |
| --- | --- | --- | --- | --- | --- |
| 2026-09-04 08:22 | review | admin | reviewer | DRAFT → REVIEWED | sample review |
| 2026-09-04 08:22 | approve | admin | approver | REVIEWED → APPROVED | sample approval |

| injector | recommended [bbl/d] | implemented [bbl/d] |
| --- | --- | --- |
| I-1 | 3,612 | — |
| I-2 | 2,722 | — |
| I-3 | 0 | — |
| I-4 | 0 | — |
| I-5 | 2,695 | — |

# Reproducibility

| item | value |
| --- | --- |
| run id | f1f43bbedd07 |
| data snapshot hash | f7fda78b31f7813fabf5d9e653ed8508b3f642bb382c1ebbbf405b4a1ddc592e |
| config hash | 7f90028e1dc16a82f17887a8f06f66883030895f1fc53860299672b9c6576ee0 |
| code version | cbb7eec |
| seed | 232413243 |
| engine runtime | 22.3 s |
| windows | 2010-01-01 → 2019-12-01 (full history) |
| application | Waterflood Optimizer 0.5.0 |